


Newcamd Cardsharing How-to

In this guide is assumed for the use cardsharing by LAN (local Area Network)

therefore not by Internet.

2 files must be changed for the cardsharing to be operate correct.

This concerns the following 2 files:

1. Cardserv.cfg

2. Newcamd.conf

These 2 files are to find in the following directory: /var/tuxbox/config

As an example we will use here 3 dreamboxes to be there of which 1 if server will serve and the other 2 dreamboxes as a customer.

We give the server and the customer here uses a name and ip-adres that also in this guide go become:


Server:
Client 1:
Client 2:

Name
server
Box-1
Box-2

password

Pwdbox-1
Pwdbox-2

Ip-address
192.168.0.97
192.168.0.100
192.168.0.42

Cardserv.cfg on the server

DESKEY= 01 02 03 04 05 06 07 08 09 10 11 12 13 14

SERVER_NAME= server

DEBUG_PORT= 14000

ADMIN_PORT= 14001

ADMIN_PASSWORD= secret 00 00 00 00 00 00 00 00 00 00 00 00 00 00

USER= local local lan dreambox 12000

USER= Box-1 pwdbox-1 lan 192.168.0.100 12000 server

USER= Box-2 pwdbox-2 lan 192.168.0.42 12000 server

DEBUG_INIT= yes

DEBUG_NET= yes

DEBUG_ECM= yes

DEBUG_EMM= yes

SERIAL_PORT= 0

TCP_PORT= 10001

EMM-G= no

EMM-S= no

EMM-U= no

SERIAL_PORT= 1

TCP_PORT= 10000

EMM-G= no

EMM-S= no

EMM-U= no

SERIAL_PORT= 2

TCP_PORT= 10002

EMM-G= no

EMM-S= no

EMM-U= no

Just as a short explanation what we have now done in this file: 2 users have been made to for 'client-1'and 'client-2', this customer has got the name 'Box-1' and 'Box-2'. These both boxes make a connection by means of port 12000 to the server who also the name has 'server'.

Newcamd.conf on client 1

DEMUX_API= 2

BOX_TYPE= DM500

CONTROL_WORD_DEVICE= CA

CA_API= 3

GTX_OUTPUT_MODE= 16

PMT_UPDATE_WATCH= no

EMU= yes

PREFER_EMU= yes

DISABLE_CAM= yes

CAM_CASYS= 17

AU= yes

SHOW_ALL_CA_SYSTEMS= no

OSD= none

DEBUG_ECM= yes

DEBUG_EMM= yes

DEBUG_CWS= yes

DEBUG_CAM= yes

DEBUG_CAM_HEX= no

SHOW_CW= no

SHOW_PAT= no

SHOW_PMT= no

#SHOW_ECM= yes

SHOW_CAT= no

SHOW_EMM= yes

EMM_REPORT_WRONG_SIG= no

OSD_WAIT_TIME= 1000

DEFAULT_CW_DELAY= 500

RELOAD_CONFIG_ON_ZAP= yes

RELOAD_KEYLIST_ON_ZAP= yes

CWS= 192.168.0.97 10001 Box-1 pwdbox-1 01 02 03 04 05 06 07 08 09 10 11 12 13 14 server

CWS_CONNECT_TIMEOUT= 20

CWS_INCOMING_PORT= 12001

CWS_DEBUG_PORT= 12001

ADMIN_PASSWORD= test 00 00 00 00 00 00 00 00 00 00 00 00 00 00

Newcamd.conf on client 2

DEMUX_API= 2

BOX_TYPE= DM7020

CONTROL_WORD_DEVICE= CA

CA_API= 3

GTX_OUTPUT_MODE= 16

PMT_UPDATE_WATCH= no

EMU= yes

PREFER_EMU= yes

DISABLE_CAM= yes

CAM_CASYS= 17

AU= yes

SHOW_ALL_CA_SYSTEMS= no

OSD= none

DEBUG_ECM= yes

DEBUG_EMM= yes

DEBUG_CWS= yes

DEBUG_CAM= yes

DEBUG_CAM_HEX= no

SHOW_CW= no

SHOW_PAT= no

SHOW_PMT= no

#SHOW_ECM= yes

SHOW_CAT= no

SHOW_EMM= yes

EMM_REPORT_WRONG_SIG= no

OSD_WAIT_TIME= 1000

DEFAULT_CW_DELAY= 500

RELOAD_CONFIG_ON_ZAP= yes

RELOAD_KEYLIST_ON_ZAP= yes

CWS= 192.168.0.97 10001 Box-2 pwdbox-2 01 02 03 04 05 06 07 08 09 10 11 12 13 14 server

CWS_CONNECT_TIMEOUT= 20

CWS_INCOMING_PORT= 12001

CWS_DEBUG_PORT= 12001

ADMIN_PASSWORD= test 00 00 00 00 00 00 00 00 00 00 00 00 00 00

As last rest still to put back the file on the corret locations:

Cardserv.cfg on the server it is placed: /var/tuxbox/config

Newcamd.conf on every customer it is placed: /var/tuxbox/config.

However, you pay attention the correct newcamd.conf on the correct customer place on that.

