F1UP0001 and F5D7231 PRINT SERVER set up

An alternative to using the Belkin software which seems to be problematic the Print server on the new router and the wireless print server can be set up manually using TCP/IP. 

The only setting that needs to be known before running through the following set-up is the IP address of the print server. In the case of the F5D7231 it will be the normal 192.168.2.1 but obviously in the case of the F1UP0001 you will need to look in the DHCP client list of the router.

1. Open the printer properties and click on Add Port, select TCP/IP port.

[image: image1.jpg]pson Stylus Photo 70

T [ ©
| T

g Epson Siylus Photo 870 ES

2|

ciceSetings | Utites |
P | Advareed |

C/P 2 (Copy 1)

Pt tothe folowing por(s). Documen
checked port

s wil pint o the fs free

Pot__[ Descrplion |

Priter

LPTT Piter Port
O LPT2 Printer Port
O LPT3  Printer Port
O com2 Serial Port
O coM3: Serial Port
O coM4: Serial Port
O coMe: Serial Port

AddPort. Deletc

I~ Enable bidrestional support
T~ Enable printer pooling

1

Avalable part types:

Bekirport
Eluetocth Priner Port
Loca port





2. The wizard appears, click Next
[image: image2.jpg]Welcome to the Add Standard
TCP/IP Printer Port Wizard

Yo uss this wizaid o add a port or 3 network piiter.

Belore continuing be sure that
1. The device is tmed an.
2 The network is connected and configured.

To continue, cick Nex.

=





3.  Enter the IP address of the Print server / Router in the space, ‘Printer name’. Port name will set itself.
[image: image3.jpg][Add Standard TCP/IP Printer Port Wizard

Add Port
For which device do you wart to add 3 port?

Ente the Pinter Name o1 [P address, and a port name for the desired device.

Fiinter Name or P Address:  [19216823 |
Port Name: [P1s216823

ok Cance





4.  Select the default ‘Generic Network card’ and click next.
[image: image4.jpg]Add Standard TCP/IP Printer Port Wizard

Additional Port Information Required
The device could nat be idertiied

The device i nat found on the netwark, Be sure that

1. The device s tumed on
2 The network is connected.

3. The device s properly configured.

4. The addess on the previous page is corect

1 youtink the s i not corect, click Back to etu o the previous page. Then corect
the address and perform anather seaich on the netuork. |f you are sure the address is corect,
selectthe device lype below.

Device Type:
& Standard

CoCuwstom Setinge

Bk Cance





5.  If the printer you are installing is the 2nd printer, the port number that the printer is printing to needs to be altered. Select the option ‘Custom’ and click on ‘Settings’
[image: image5.jpg]Add Standard TCP/IP Printe

Additional Port Information Required
The device could nat be idertiied

1. The device s tumed on

2 The netwak s cormeted P
3. The device is propety confiqured.
4 The addess onthe previous pageis  ForName:
1 o tink the adeess s ot corect e pryer Namme o P Adess:
the addess and pefom another seach ¢ e 1o o P Add 1s216825
selct the device type belo o
Device Type——————————— © Ran R
€ Standad [Feneic Netwark ad
~Raw Selings

© Cuson Setings Port Nuber EL

- LPR Selings
e Hame:
| | FiieRByeCoutngEnsbld

-1~ SNMP Status Enabled

Community Nane:— [pable
SHHP Devioe Indes: 7

Cance





[image: image6.jpg]Printer Port Wizard

Completing the Add Standard
TCP/IP Printer Port Wizard

Yo have selected a port withthe following cheracteristics.

SNMP: No
Frotocol  RAW, Fort 3100
Device: 19216825

PotName:  IP_19216825
Adspler Type:

To completethis wizard,clck Frish

<oack -





Printer 2 will print to port number 9101





When installing the second printer the port only needs to be ‘configured’ not ‘Added’. So in the printer properties the option ‘configure’ is selected and the port number is changed.








