Preliminary


relook-3000 Flash Upgrade Procedures

By Michael H. Kim

V1.0.0 (Nov 17, 2003)

0. What’s New in Flash-based relook-3000
In the new, flash-based relook-3000, followings are different from the previous, HDD-based one.
(1) relook’s flash memory size has increased to 8MB to accommodate Linux kernel, system software, application software, and DBs.  Since all software resides on flash memory, it works similar to 2-CI slot receivers even without HDD installed.

(2) It is fully functional without an HDD with the exception of EPG and HDD-dependent operations such as trick plays for live and recordings.

(3) User can select his/her preferable HDD in the market and install it by himself/herself.

(4) HDD formatting is supported.

A. Select “FORMAT” from User’s Option in the Main Menu.

B. relook-3000 will be restarted and its HDD will be formatted and re-initialized.

(5) Exchange HDD with your friend.

A. Select “EXCHANGE” from User’s Option in the Main Menu.

B. HDD will be software-protected and current DB will be maintained in the HDD.

C. If you install this HDD into other relooks, the DB maintained in HDD will be used.

(6) When updating flash image, both the DB in flash memory and in HDD are updated and synchronized.

1. Getting Started

Warning !!!

uboot — the boot-loader of flash-based relook-3000 — is the most sensitive part and have to be upgraded with caution.  During flash upgrading, relook should not be turned off.  In case of uboot upgrade failure, please contact with hansoo@dgstation.co.kr or MSN at viscom60@msn.com.
Each version/release of flash image has naming convention as FlashImages_v.1.1.0_R1.zip and contains all or some of followings:

	Image
	Descriptions
	HDD Auto Format

	usb_uboot
	· Boot loader image for relook-3000.
	

	usb_kernel
	· Linux kernel image for relook-3000.
	

	usb_root
	· Application software image for relook-3000.
· Usually, incremental upgrade of relook-3000 is done by downloading/replacing this image.
	

	usb_db
	· Full DB image for relook-3000.
· Downloading this image replaces and initializes the DB previously on flash and HDD.
	Yes

	usb_usrdb
	· Incremental db image for relook-3000.
· Download this image performs incremental DB updates such as new TPs, new multi-languages, etc.
	

	usb_flash_entire
	· Entire flash image for relook-3000.
	Yes

	usb_flash_without_boot
	· Entire flash image for relook-3000 without uboot.
	Yes


In order to download new flash images, you should install a download utility “FlashTools” onto your PC.
The “FlashTools” is an integrated utility for downloading new flash images using USB or serial connection.  It is preferred to use the USB connection.  In case of serial downloading, you need special images which are not released officially.

2. Installing Flash Download Utility “FlashTools”
1. Double click the “Flash_Installer.exe” icon and follow the installation procedures.
2. A new folder (Flash_Tools) will be created under the “Program Files” folder.  The download utility and device drivers will be installed in the “Flash_Tools” folder.
3. Installing USB Driver for Flash-based relook-3000
Before invoking the flash download utility “FlashTools”, your PC have to recognize relook-3000 first.  Please follow below instructions carefully.

Remark 1 : Before installing the USB driver for relook-3000 onto your PC, you have to turn your relook-3000 off.
1. Connect the USB cable between USB port of PC and USB port of relook-3000.
2. While pressing and holding the Power button on the front of the relook-3000, press Up button on the front for a second (during when TV displays “Welcome Image”) then release the Up then the Power buttons.  The TV screen changes from “Welcome Image” to the famous “Penguin Image” and your PC detects relook-3000 as a new USB hardware and invokes the Driver Installation Wizard automatically.
[image: image1.png]Found New Hardware Wizard

‘Welcome to the Found New
Hardware Wizard

yournstal

*), If your hardware came with an installation CD
<2 or floppy disk. insert it now.




3. Please follow the instructions of the Driver Installation Wizard.

[image: image2.png]Found New Hardware Wizard

‘Welcome to the Found New
Hardware Wizard

This wizard helps you nstall

1If your hardware came with an installation CD
<2 or floppy disk. insert it now.





4. Designate the folder where the file “DGUSB.INF” is saved.  The “Flash_Toos” folder under the “Program Files” folder is the default folder.
[image: image3.png]Found New Hardware Wizard

Please choose your search and installation options.

&

Search forthe best diver i these lacatins.

Use the check boses below o it or expand the default search, which includes local

paths and rerovable medis. The best diver found wil be nstalled

[ Search removable media (floppy, CD-ROM...)

Incude this localion i the search:

E#i86ilang v| [CBrowse

O Den'tssarch. | will choose the diiver to nstall

Chaase this optian to select the device civer fiom a s, Windows does not quarartee that

the dhiver you chaose wil be the best malch for your hardware.





5. Click on “OK” on the pop-up window and “Next>” on the Installation Wizard.

[image: image4.png]Please choose your search and installation options.

| Browse For Folder

@ Search fof - select the folder that contains hivers for your harduware.

Use e of
o and
@ owieer
05 (& & wyoocuments
® 2 My Computer
o | ® 2w Natwor pces
© amage
© 5 Unused estop shrtuts

Choose ——
the diver o

To view any subfolders, click. plus sign above.





6. Please wait until the wizard finish the driver installation.  If the wizard displays an alert message window relating with USB compatibility test, please click “Continue.”
[image: image5.png]Please wait while the wizard installs the software.

@, DBDSBUSBFORVESTA

2





7. Click “Finish” on the pop-up screen to finish the installation.  Make sure that the front of relook-3000 displays “USB”.

[image: image6.png]Found New Hardware Wizard

Installing Completed

Instaling completed this harchn

Firish1





4. Downloading relook-3000 Flash Images
In order to download flash images, you have to make your relook-3000 into USB Connection mode.  This is not difficult but requires some exercises to be familiarized with.
Before Starting

Before you start the new flash image download, please confirm the current version of application software (Main Menu ( About ( System Info).  Also make sure that all the S/W upgrade utilities are completely installed on your PC.
Remark 1 : Before upgrading your relook-3000, you have to turn the relook-3000 off.
Remark 2 : If this is your first time to install the driver and download utility, you don’t need to turn off the relook-3000.  “FlashTools" will be automatically invoked after installed and the front of the relook-3000 will display “USB.”  If you want to turn off the relook-3000 forcedly, please press “OK” and “Power” button on the front panel together.

How to Download New Flash Images
1. Turn your relook-3000 off then connect USB cable between relook-3000 and PC.

2. Run the flash download utility program “Flash Tools” on your PC.
[image: image7.png]=lolx]

FuasuTooL ™
O SERIAL usB @
USE]
COM Port

PorTNo. [ 7  DamBr [ 8
Baup RaTe 115200  PaRimY none
FLow CoNTROL [ nane

FILE
T ® Fue

STATUS
ProGRESS 0 min. 0 sec. / 0 min. 0 sec.
r %

MESSAGE.

® Reser





3. Click “USB” button on the upper part of “FlashTools” window.  You can also see the status message “waiting” in the MESSAGE window.
[image: image8.png]FuasuTooL ™

© SERIAL Ouse

FILE

USB
DOWNLOAD |

———— PROGRESS

DownLoAD :

r %

ERASING

r %

WRITING

r %
MESSAGE :

e





4. While pressing and holding the Power button on the front of the relook-3000, press Up button on the front for a second (during when TV displays “Welcome Image”) then release the Up then the Power buttons.  The TV screen changes from “Welcome Image” to the famous “Penguin Image.”  
[image: image9.png]Home €ntertainment




 [image: image10.png]-Boot 0.3.0 (How 10 2603 - 20:02:00)

Cannecting 10 ta O




The front of the relook-3000 displays “USB” and the “FlashTools” displays “USB is connected” in its message window.

[image: image11.png]FuasuTooL ™

© SERIAL

Ouse

FILE

[CaDocuments and SetfingstAdminisiratoriby Documentsidre [

(o Fue

)

USB
DOWNLOAD

PROGRESS
DownLoAD
r %
ERASING
r %
WRITING
r %

Viting n Flash |
Complele





5. Click the “FILE” button and select a flash image to download.  You can select and download the full flash images such as “usb_flash_entire” or “usb_flash_without_boot” or each images one after another.  In the later case, you have to download the “usb_uboot” first then “usb_kernel”, “usb_root”, “usb_db”, “usb_usrdb”, etc.
[image: image12.png]-

FuasuTooL ™
© SERIAL Ouse
x|
e
. U¢ [537.0.2 Official Release ] DGMans. Fuie
DOWN |5 201 Jash
DM ED @ jpload 031023 S Ve TR
|Z1DGMan 1.3 (031107) (1 Upload 031024 37|: 8.00MB
|Z1DGMan 1.3 (031108) Upload 031027
| )
D
T EAD:  [Image Fle(img) 2 BTy omi
Y

Walting





6. Press the “DOWN” button and wait until the progress bar for Download, Erasing and Writing reaches 100%.
[image: image13.png]FuasuTooL ™

© SERIAL

. uUsB
DOWNLOAD

Ouse

FILE

[CaDocuments and SetfingstAdminisiratoriby Documentsidre [

PROGRESS
DownLoAD

T
ERASING

%

T
WRITING

%

%

Complete
UsB s connected
ush_flash_entiremg is

® Fue

© Down



 [image: image14.png]FuasuTooL ™

© SERIAL

USB.

DOWNLOAD

Ouse @
FILE
[CADocuments and SeffingsiAdministratority Documentsi @21 (@ Fue |
PROGRESS
DownLoad :
I 66%
ErasinG [N
T 0%
Warmna : ——
[ % Down )
MESSAGE

Complete
UsB s connected




 [image: image15.png]FLasnTooL

© SERIAL

USB.

DOWNLOAD

Ouse

FILE
[CADacuments and SefingsWdministralory Documents\gre

PROGRESS
DownLoad :
I 100%
ErasiN
I 74%
WWRITING N R
[ % Down )
s selected

Download Start




 [image: image16.png]FuasuTooL ™

© SERIAL

USB.
DOWNLOAD

Ouse
FILE
[CADacuments and Settings\Administratoriby Documents\gY;
PROGRESS
DownLoap :
I 100%
ERasiNG
I 100%
WRITING ©
| 50% [k
MESSAGE :

Download Start
Erase Flash

® Down



 

The TV screen also shows the progress of flash upgrading.

[image: image17.png]| ool 0.9.0 (o 10 2003 - 20:02:00)

Camnee g U130 4o FC.
checking recelved inage.
ot entire lach (mge.
Arane D 000000 - OXTFTCET




 [image: image18.png]| oot 0.9.0 (o 10 7003 - 20:02:00)

CamneeLing G0 G PO
ahecking Fecolved Inage. .
ot entire fiach inage

race Oxr 000000 - DRFOCOT

Fraved 147 roctors
urite 0 Flogh.




7. After completing, the “FlashTools” displays “Complete” in its message window and the front displays “donE.”  Now press Power button on the front or the RCU to restart your relook-3000.
[image: image19.png]FLasnTooL

© SERIAL Ouse @

FILE

[CADacuments and SefingsWdministralory Documents\gre

PROGRESS

Downoad

T %

ErasiN

T %

Warmna :

f 0% © Down

uUsB 1
DOWNLOAD L Ol

MESSAGE :
Erase Flash
Wiiting in Flash





[image: image20.png]-Boot 0.3.0 (ow 102603 ~ 20:02:00>

Camnec Ling 10 ta PO
Ghecking Fecolved Inags. .
ot entie finch invge.
rave Ox( 800000 - OXTFICET

Fraved 142 rooiors

Completea




If you have any technical question on installing device driver and flash download utility or upgrading the relook-3000 application software, and have any comment on this manual, please contact with hansoo@dgstation.co.kr or MSN account viscom60@msn.com.
Instruction Manual v1.0.1 - FlashTools                                                                     4

